# Model 7110 PCI interface map

PCI bus IDSEL#

On the Midas board the following IDSEL is generated.

| PCI device | IDSEL # | PCI address for config cycle |
| --- | --- | --- |
| PMC # 1 | AD[16] | 0x00010xxx |
| PMC #2 | AD[17] | 0x00020xxx |
| VME-PCI bridge | AD[31] | 0x80000xxx |

PCI addressing

There are three types of PCI-defined address space:

Configuration space	Configuration space is a contiguous 256 x 8-bit set of addresses dedicated to each “slot” or “stub” on the bus. Decoding C_BE/(3-0) determines if a PCI cycle is intended to access configuration register space.

Memory space

I/O space

# PCI 9080 - PCI interface

The PCI 9080 chip provides several internal registers, allowing for maximum flexibility in bus interface design and performance. The register types are accessible from both the PCI and Local Buses, including the following:

- PCI Configuration registers

- Local Configuration registers

- Mailbox registers

- Doorbell registers

- DMA registers

- Messaging queue registers (I2O)

## Register access from the PCI bus

The PCI 9080 PCI configuration registers can be accessed from the PCI bus with a type 0 configuration cycle.

The PCI 9080 internal registers can be accessed by a memory cycle, with the PCI bus address that matches the base address specified in the PCI base address 0 for the memory mapped configuration register of the PCI 9080. They can also be accessed by an I/O cycle, with the PCI bus address matching the base address specified in the PCI base address 1 for the I/O mapped configuration register of the PCI 9080.

All PCI read or write accesses to the PCI 9080 can be byte, word or lword accesses.

## Register access from the local bus

The local processor can access all the internal registers of the PCI 9080 through either internal or external access decide logic.

All local read and write accesses to the PCI 9080 registers can be bye, word or lword accesses.

## Direct Master Operation

The PCI 9080 supports direct master access of the PCI bus by the local processor. In a Direct Master Write, the local processor (Master) writes data to the PCI (Slave). In a Direct Master Read, the local processor (Master) reads data from the PCI (Slave) Master mode must be enabled in the PCI Command Register. Five registers are used to define Local-to-PCI access:

- Range (DMRR – 0x9C)

- Local Base Address for Direct Master to PCI Memory Register (DMLBAM – 0xA0)

- Local Base address for Direct Master to PCI IO/CFG Register (DMLBAI – 0xA4)

- PCI Configuration Address Register for Direct Master to PCI IO/CFG (DMCFGA – 0xAC)

- PCI Base Address (DMPBAM – 0xA8)

The range register specifies the local address bits to use for decoding a local-to-PCI access. The local processor can perform only memory cycles. Therefor the local base address for the Direct Master to PCI memory register is used to decode an access to PCI memory space. The local base address for Direct Master to PCI IO/CFG register is used to decode an access to PCI I/O space or PCI configuration space.

The local processor can read or write to the PCI memory. The PCI 9080 converts the local Read/Write access. The local address space starts from the Direct Master Local Base Address up to the range. Remap (PCI base address) defines the PCI starting address.

### IO/CFG access

When a Local Direct Master I/O access to the PCI bus is made, the configuration bit of the PCI configuration address register determines if I/O or configuration access is made to the PCI bus.

If the configuration enable bit is clear, a single I/O access is made to the PCI bus. The Local Address, remapped decode address bits and the local byte enables are encoded to provide the address and is output with an I/O read or write command during the PCI address cycle.

If the configuration enable bit is set, a CFG access is made to the PCI bus. In addition to enabling the configuration bit of DMCFGA, the user must provide all register information. The register number or the device number must be modified and a new CFG read/write cycle must be performed before other registers or devices can be accessed.

Note: this clearly states that after programming the registers, a read/write access must be performed to the local bus IO space.

## PCI Configuration Registers

The following table shows the PCI configuration register definition for the Model 7110. This is a standard layout for the configuration registers as defined in the PCI specification. The only differences are the local address offsets. These are adapted to the C44 addresses.

|  | PCI configuration registers |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| C44 local offset | D31-D24 | D23-D16 | D15-D08 | D07-D00 | PCI CFG register Address |
| 0x0000 | Device ID |  | Vendor ID |  | 0x0000 |
| 0x0001 | Status |  | Command |  | 0x0004 |
| 0x0002 | Class Code |  |  | Rev ID | 0x0008 |
| 0x0003 | BIST | Header type | Latency timer | Cache Line Size | 0x000C |
| 0x0004 | PCI Base Address for Memory Mapped Runtime Registers |  |  |  | 0x0010 |
| 0x0005 | PCI Base Address for I/O Mapped Runtime Registers |  |  |  | 0x0014 |
| 0x0006 | PCI Base Address for Local Address Space 0 |  |  |  | 0x0018 |
| 0x0007 | PCI Base Address for Local Address Space 1 |  |  |  | 0x001C |
| 0x0008 | Reserved |  |  |  | 0x0020 |
| 0x0009 | Reserved |  |  |  | 0x0024 |
| 0x000A | Cardbus CIS pointer (Not Supported) |  |  |  | 0x0028 |
| 0x000B | Subsystem ID |  | Subsystem Vendor ID |  | 0x002C |
| 0x000C | PCI Base Address to Local Expansion ROM |  |  |  | 0x0030 |
| 0x000D | Reserved |  |  |  | 0x0034 |
| 0x000E | Reserved |  |  |  | 0x0038 |
| 0x000F | Max_Lat | Min_Gnt | Interrupt Pin | Interrupt Line | 0x003C |

The PCI 9080 PCI configuration registers can be accessed from the PCI bus with a type 0 configuration cycle.

The PCI 9080 internal registers can be accessed by a memory cycle, with the PCI bus address that matches the base address specified in the PCI Base Address 0 for memory mapped configuration register of the PCI 9080.

They can also be accessed by an I/O cycle with the PCI bus address matching the base address specified in the PCI Base Address 1 for the I/O mapped configuration register of the PCI 9080.

Local bus access to internal registers

The local processor can access all the internal registers of the PCI 9080 through either internal or external address decode logic.

# Universe VME-PCI interface

The Universe Control and Status Registers (UCSR) occupy 4 Kbytes of internal memory. This 4 Kbytes is logically divided into three groups.

PCI configuration space (PCICS)

Universe Device Specific Registers (UDSR)

VMEbus Control and Status Registers (VCSR)

The Universe registers are little endian.

The access mechanisms for the UCSR are different depending upon whether the register space is accessed from the PCI bus or the VMEbus.

## Register access from the PCI bus

There are two mechanisms to access the UCSR space from the PCI bus: through Configuration space or through the PCI defined base address register.

When the UCSR space is accessed as Configuration space, it means that the access is externally decoded and the Universe is notified via IDSEL# (sort of chip select). Since the register location is encoded by a 6-bit register number, only the lower 256 bytes of the UCSSR can be accessed as configuration space (this corresponds to the PCICS in the UCSR space). So only the PCI configuration registers are accessible through PCI.

To access the other UCSR register through Memory or I/O space, the base address of the PCI_BS register must first be programmed. Since in Memory or I/O space the UCSR registers are located as address offsets from the base address programmed into the BS field in the PCI_BS register.

### Memory or I/O access

The UCSR space can be configured as Memory or I/O space at power up. An external pin determines if the UCSR space is accessible as Memory or I/O space. The user is free to locate the UCSR register anywhere in the 32-bit Memory or I/O space.

## Register access from the VMEbus

There are two mechanisms to access the UCSR space from the VMEbus. One method uses the VMEbus Register Access Image (VRAI) which allows the user to put the UCSR in an A16, A24 or A32 address space. The VRAI approach is used in the PCI project. The other way to access the UCSR is as CR/CSR space, where each slot in the VMEbus system is assigned 512 Kbytes of CR/CSR space.

## Universe Interfaces

### VMEbus Interface

VMEbus slave	The Universe acts like a VME slave when accessed from the VMEbus.

VMEbus master	The Universe becomes a VMEbus master when the VME master interface is internally requested by the PCI Bus Slave Channel, the DMA Channel or the Interrupt channel.

### PCIbus Interface

PCI slave	The Universe acts like a PCI slave when accessed from the PCI bus.

PCI master	The Universe becomes PCI master when the PCI master interface is internally requested by the VME slave Channel or the DMA channel.

## Slave Image programming

The Universe recognizes two types of access on its bus interface: access destined for the other bus, and accesses decoded for its own register space.

### VME slave images

The Universe accepts accesses from the VMEbus within specific programmed slave images. Each VMEbus slave image opens a window to the resources of the PCI bus. The Universe allows four different VME slave images to be programmed.

Vmebus fields for VME slave images

Decoding for VMEbus access is based on the address. Before responding to an external VMEbus master, the address must lie in the window defined by base and bound addresses

| Field | Register bits | description |
| --- | --- | --- |
| Base | BS[31:12] | multiples of 4 or 64 Kbytes |
| Bound | BD[31:12] | multiples of 4 or 64 Kbytes |
| Address space | VAS | A16,A24,A32,User 1,User 2 |
| Mode | SUPER | supervisor |
| Type | PGM | program and/or data |

PCI bus fields for VME Slave images

The PCI bus fields specify how the VMEbus transaction is mapped to the appropriate PCI bus transaction. The translation offset allows the user to translate the VMEbus address to a different address on the PCI bus.

| Field | Register bits | description |
| --- | --- | --- |
| Translation offset | TO[31:12] | Offset VMEbus slave address to a selected PCI address |
| Address space | LAS | Memory, I/O, configuration |
| RMW | LLRMW | RMW enable bit |

Control fields for VME slave images

The control fields allow the user to enable a VMEbus slave image, as well as specify how reads and writes will be processed.

| Field | Register bits | description |
| --- | --- | --- |
| Image enable | EN | Enable bit |
| Posted write | PWEN | Posted write enable bit |
| Prefetched read | PREN | Prefetched read enable bit |
| Enable PCI D64 | LD64EN | Enables 64-bit PCI bus transactions |

### PCI Bus Slave Images

PCI bus fields for PCI slave images

Decoding for VMEbus access is based on the address. Before responding to an external VMEbus master, the address must lie in the window defined by base and bound addresses

| Field | Register bits | description |
| --- | --- | --- |
| Base | BS[31:12] | multiples of 4 or 64 Kbytes |
| Bound | BD[31:12] | multiples of 4 or 64 Kbytes |
| Address space | LAS | Memory, I/O, configuration |

VMEbus fields for PCI Slave images

The PCI bus fields specify how the VMEbus transaction is mapped to the appropriate PCI bus transaction. The translation offset allows the user to translate the VMEbus address to a different address on the PCI bus.

| Field | Register bits | Description |
| --- | --- | --- |
| Translation offset | TO[31:12] | Translates address supplied by PCI master to a specified VMEbus address |
| Maximum data width | VDW | 8,16,32 or 64 bits |
| Address space | VAS | A16,A24,A32, CR/CSR, User 1,User 2 |
| Mode | SUPER | supervisor |
| Type | PGM | program or data |
| Cycle | VCT | Single or block |

Control fields for PCI bus slave images

The control fields allow the user to enable a VMEbus slave image, as well as specify how reads and writes will be processed.

| Field | Register bits | Description |
| --- | --- | --- |
| Image enable | EN | Enable bit |
| Posted write | PWEN | Posted write enable bit |