# Connectoren

## PEM input connector

Input at reset (Comm 4 and 5)

| Pin | signal | Pin | signal |
| --- | --- | --- | --- |
| 1 | CD0 | 2 | GND |
| 3 | CD1 | 4 | GND |
| 5 | CD2 | 6 | GND |
| 7 | CD3 | 8 | GND |
| 9 | CD4 | 10 | GND |
| 11 | CD5 | 12 | GND |
| 13 | CD6 | 14 | GND |
| 15 | CD7 | 16 | GND |
| 17 | CREQ | 18 | GND |
| 19 | CACK | 20 | POL_IN* |
| 21 | CSTRB | 22 | GND |
| 23 | CRDY | 24 | GND |
| 25 | DIRIN | 26 | DIROUT |

Output at reset (Comm 1 and 2)

| Pin | signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | CD0 | 2 | GND |
| 3 | CD1 | 4 | GND |
| 5 | CD2 | 6 | GND |
| 7 | CD3 | 8 | GND |
| 9 | CD4 | 10 | GND |
| 11 | CD5 | 12 | GND |
| 13 | CD6 | 14 | GND |
| 15 | CD7 | 16 | GND |
| 17 | CREQ | 18 | GND |
| 19 | CACK | 20 | GND |
| 21 | CSTRB | 22 | GND |
| 23 | CRDY | 24 | POL_OUT* |
| 25 | DIRIN | 26 | DIROUT |

Note : * pin is cut on male connector

## Correlator board connector

| Pin | A | B | C |
| --- | --- | --- | --- |
| 1 | C0D0 | GND | C5D0 |
| 2 | C0D1 | GND | C5D1 |
| 3 | C0D2 | GND | C5D2 |
| 4 | C0D3 | GND | C5D3 |
| 5 | GND | GND | GND |
| 6 | C0D4 | GND | C5D4 |
| 7 | C0D5 | GND | C5D5 |
| 8 | C0D6 | GND | C5D6 |
| 9 | C0D7 | GND | C5D7 |
| 10 | GND | GND | GND |
| 11 | CREQ0 | GND | CREQ5 |
| 12 | CACK0 | GND | CACK5 |
| 13 | GND | GND | GND |
| 14 | CSTRB0 | GND | CSTRB5 |
| 15 | GND | GND | GND |
| 16 | CREADY0 | GND | CREADY5 |

# Cables

## Standard cable

| PEM | Correlator | Description |
| --- | --- | --- |
| 1 | A1 | CD0 |
| 2 | B1 | GND |
| 3 | A2 | CD1 |
| 4 | B2 | GND |
| 5 | A3 | CD2 |
| 6 | B3 | GND |
| 7 | A4 | CD3 |
| 8 | B4 | GND |
| 9 | A6 | CD4 |
| 10 | B6 | GND |
| 11 | A7 | CD5 |
| 12 | B7 | GND |
| 13 | A8 | CD6 |
| 14 | B8 | GND |
| 15 | A9 | CD7 |
| 16 | B9 | GND |
| 17 | A11 | CREQ |
| 19 | A12 | CACK |
| 21 | A14 | CSTRB |
| 23 | A16 | CRDY |

Note 1: PEM pins 18,20,22,24 can also be connected to B11, B12, B14, B16, but is not required.

Note 2: the function of PEM pins 25 and 26 (DIRIN and DIROUT) is not clear. Don’t connect these pins, but leave them available for future use.

Cable length: max 75 cm

## Special cable

| PEM | Correlator | Description |
| --- | --- | --- |
| 1 | C1 | CD0 |
| 2 | B1 | GND |
| 3 | C2 | CD1 |
| 4 | B2 | GND |
| 5 | C3 | CD2 |
| 6 | B3 | GND |
| 7 | C4 | CD3 |
| 8 | B4 | GND |
| 9 | C6 | CD4 |
| 10 | B6 | GND |
| 11 | C7 | CD5 |
| 12 | B7 | GND |
| 13 | C8 | CD6 |
| 14 | B8 | GND |
| 15 | C9 | CD7 |
| 16 | B9 | GND |
| 17 | C11 | CREQ |
| 19 | C12 | CACK |
| 21 | C14 | CSTRB |
| 23 | C16 | CRDY |

Note 1: PEM pins 18,20,22,24 can also be connected to B11, B12, B14, B16, but is not required.

Note 2: the function of PEM pins 25 and 26 (DIRIN and DIROUT) is not clear. Don’t connect these pins, but leave them available for future use.