# Post Correlator Integrator
Requirements

- Fit within the correlator design

- Receive control messages from the Correlator Control Computer (CCC)

- Interpret control messages to determine which of a defined set of processes is to be performed.

- Receive multiple channels of correlator data

- Store and re-order the data as required

- Accumulate data as required

- Perform DSP algorithms such as complex multiplication and Fourier transformation as required.

- Report status information to the CCC on request.

- Pass the product of processing to TBD output port or removable storage media.

- The storage media must have a large capacity to store several hours of observations.

# Post Correlator Integrator
Hardware

## Monaco board from Spectrum Signal Processing

## Features

- four TMS320C6701 processing nodes

- shared access to a 132 MB/sec PMC module via the Spectrum Hurricane chip

- shared access to 512K x 32 bit of fast SRAM

- VME A24 slave interface to the ‘C6x Host Port Interface

- two 400 MB/sec Processor Expansion Modules

Each processing node

- one 167 MHz TMS320C6701 floating point DSP

- 128K x 32 bit of SBSRAM

- 4M x 32 bit of SDRAM

# Post Corrlator Integrator
rack

# Post Correlator Integrator
Software

- receive data from correlator board

- process data

- write data to tape

- interface to RT-system

# Post Correlator Integrator
Results

## Input data rate

- approx. 15 MB/sec over 20 MB/sec C4x COMM ports

## Output data rate

- Limited by tape drive to 1.4 MB/sec

- possible up to 30 MB/sec

## Processing

- TMS320C6701 DSP theoretically 1 GFLOPS

- TMS320C40 ‘only’ 20 MIPS

# Post Correlator Integrator
work to do

- Test correlator board data input

- Correctly implement write handler task

- Define and implement processing applications

- RT-system synchronization of PCInt tasks and Correlator board tasks